CPE301 – SPRING 2019

# Design Assignment 5

Student Name: Do Le

Student #: 2001183621

Student Email: led2@unlv.nevada.edu

Primary Github address: https://github.com/DoVietLe

Directory: https://github.com/DoVietLe/assignments

The goal of the assignment is to develop the above code to do the following:

1. Using the Multi-function shield, using a custom serial transfer protocol display the temperature sensor value (int only) of LM34/35 on to the seven-segment display.

2. Using the Multi-function shield, using the SPI protocol display the temperature sensor value (int only) of LM34/35 on to the seven-segment display.

3. Using one-wire interface display the temperature sensor for DS18B20 on the UART terminal.

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Components

• XPlained Mini Board

- ATmega328PB

- Programmer

• Atmel Studios

- Compiler

- Simulator

• LM35 Temperature Sensor

• DS18B20 Temperature Sensor

• Arduino Shield

- Seven-Segment Displays

Block Diagram

PC4 used as temperature analog input.

PB0 used as custom serial output.

PB3 used as SPI Master Out.

PD4 used as latch clock.

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

Task 1 Code:

#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/delay.h>

#define DDR_SHIFT DDRB

#define PORT_SHIFT PORTB

#define DDR_CLK DDRD

#define PORT_CLK PORTD

#define PIN_SCLK 7

#define PIN_LCLK 4

// Values for the seven-segment (common cathode configuration).

const uint8_t sevenSegmentCode[] = {

0x03,

0x9F,

0x25,

0x0D,

0x99,

0x49,

0x41,

0x1F,

0x01,

0x09

};

// 'Custom Serial Transfer Protocol' functions.

void initializePort() {

// Initializes the direction.

DDR_SHIFT = 0x01;							// Sets lower most bit as output.

DDR_CLK = (1<<PIN_SCLK)|(1<<PIN_LCLK);		// Sets clock pin as output.

// Initializes output.

PORT_SHIFT = 0x00;

PORT_CLK = 0x00;

}

void clkShift() {

// Sets clock input to shift register as high.

PORT_CLK |= (1<<PIN_SCLK);

//_delay_ms(1);

PORT_CLK &= ~(1<<PIN_SCLK);

}

void clkLatch() {

PORT_CLK |= (1<<PIN_LCLK);

//_delay_ms(1);

PORT_CLK &= ~(1<<PIN_LCLK);

}

void sendData(uint8_t data) {

// Places data into the port.

PORT_SHIFT = data;

// Repeatedly clocks the value into the shift register,

// then shifts the next value into the position.

for (uint8_t i = 0; i < 8; i++) {

clkShift();

PORT_SHIFT >>= 1;

}

}

void displayDigits(uint8_t digits[]) {

// For all digits in the array:

for (uint8_t i = 0; i < 4; i++) {

sendData(sevenSegmentCode[digits[i]]);	// Send the seven-segment value corresponding to the digit.

sendData(1<<(4+i));						// Send the 1-hot value corresponding to which 8-segment.

clkLatch();								// Clock in the value.

}

}

// Temperature functions.

void enableADC() {

// Sets PINC4 as input.

DDRC &= ~(1<<4);

// Sets up ADC settings.

ADMUX = 0xC4;					// Reads from ADC4. Right justified. Internal 1.1V reference.

ADCSRB = 0x00;					// Free running mode.

ADCSRA = 0b10000111;			// Enables ADC with 128 pre-scaler.

}

void disableADC() {

ADCSRA &= 0x7F;					// Disables the ADEN bit.

}

uint16_t analogRead() {

ADCSRA |= (1<<6);				// Enables ADSC to start AD conversion.

while (!(ADCSRA&(1<<4)));		// Waits until ADIF is set indicating ADC is done.

ADCSRA |= (1<<4);				// Clears ADIF flag.

return ADC;						// Returns value.

}

uint16_t analogToTemp(uint16_t t, char unit) {

// Takes analog converted number and converts it into temperature.

float convertedTemp = t*110.0/1023.0;

switch (unit) {

case 'C':

case 'c':

return convertedTemp;			// Returns temperature converted to fahrenheit.

case 'F':

case 'f':

default:

return convertedTemp*1.8 + 32;	// Returns temperature converted to celsius.

}

}

void getDigits(uint16_t value, uint8_t arr[]) {

for (uint8_t i = 0; i < 4; i++) {

arr[i] = ( (uint8_t)(value/pow(10, i)) ) % 10;

}

}

int main(void) {

// Variables.

uint16_t analogValue, temp;			// Stores the analog value and the temp.

uint8_t digits[4];					// Stores the digits.

// Sets all initial values.

initializePort();

enableADC();

while (1) {

analogValue = analogRead();

temp = analogToTemp(analogValue, 'c');

getDigits(temp, digits);

displayDigits(digits);

}

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

Task 2 Code (Modified functions from task 1 shown):

// SPI functions.

void initializeSPI() {

// Initializes ports.

DDRD |= (1<<PIND4);				// Sets latch clock pin as output.

DDRB |= (1<<PINB3)|(1<<PINB5);	// Sets MOSI pin as output.

PORTD &= ~(1<<PIND4);			// Sets clock output to zero.

//Sets SPI settings.

SPCR0 = 0b01110000;				// Enables SPI mode as master.

}

void clkLatch() {

PORTD |= (1<<PIND4);

_delay_ms(1);

PORTD &= ~(1<<PIND4);

}

void sendData(uint8_t data) {

SPDR0 = data;					// Sends data.

while ( !(SPSR0&(1<<SPIF)) );	// Waits until the data is sent.

SPSR0 |= (1<<SPIF);				// Clears the flag.

//SPCR0 |= ();

}

void displayDigits(uint8_t digits[]) {

// For all digits in the array:

for (uint8_t i = 0; i < 4; i++) {

sendData(sevenSegmentCode[digits[i]]);	// Send the seven-segment value corresponding to the digit.

sendData(1<<(4+i));						// Send the 1-hot value corresponding to which 8-segment.

clkLatch();								// Clock in the value.

}

}

Task 3 Code (Library not shown):

#define F_CPU 16000000UL

#include <avr/interrupt.h>

#include <avr/io.h>

#include <util/delay.h>

#include "ds18b20.c"

#define UBBR_VALUE 103

char myIntString[20];

// USART functions.

// Enables the UART and it's transmitter.

void enableTransmit() {

// Sets port.

DDRD |= 0x02;

// Sets baud rate.

UBRR0 = UBBR_VALUE;

// Sets UART settings.

UCSR0C = 0b00000110;			// to async mode with no parity

// 2 stop bits, and a frame of 8 bits.

UCSR0B = 0b00001000;			// Enables transmit line.

}

// Disables the transmitter on UART.

void disableTransmit() {

UCSR0B &= ~(1<<TXEN0);

}

// Sends one byte of data out the TX line.

void sendByte(uint8_t b) {

if (UCSR0B & (1<<TXEN0)) {				// Checks whether transmit is enabled.

while (!(UCSR0A & (1<<UDRE0)));		// Waits until the UDR0 register is ready.

UDR0 = b;							// Sends the byte.

}

}

void sendString(char string[]) {

int i = 0;

while (string[i] != '\0')

sendByte(string[i++]);

}

int main(void) {

// Variables.

uint16_t temp;			// Stores the analog value and the temp.

// Sets all initial values.

enableTransmit();

sei();

while (1) {

temp = ds18b20_gettemp();

snprintf(myIntString, 20, "%u\n", temp);

sendString(myIntString);

_delay_ms(500);

}

}

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

Task 1 Video

https://www.youtube.com/watch?v=GyiBURnXjiU

Task 2 Video

https://www.youtube.com/watch?v=TiWH5GA68fM

Task 3 Video

https://www.youtube.com/watch?v=w1iWFvSaYKU

- GITHUB LINK OF THIS DA

https://github.com/DoVietLe/assignments/tree/master/ESD301/LAB05

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Do Viet Le